Window Control Modbus Register Map and Operating Procedure

Purpose

This document describes the Modbus holding-register interface used by the Window Control firmware. It explains how a Modbus master application should read status values and write configuration or command values.

The firmware exposes zero-based holding-register offsets. Some PC tools also show the classic 40001 notation. For example, offset 0 is displayed as 40001, offset 100 as 40101, and so on.

Simply Modbus Master

The intended PC tool is Simply Modbus RTU/ASCII Master. Use Holding Registers only: Function Code 03 to read, Function Code 06 to write one register, and Function Code 16 to write multiple registers.

Source checked: https://simplymodbus.ca/tools-master.html

Recommended Procedure

- Connect the RS-485/serial adapter and select the correct COM port, baud rate, parity, stop bits and slave ID used by the STM32 firmware.

- To read STATUS values, select Function Code 03 - Read Holding Registers. Use the start offset from the table and the required register count.

- To write a 32-bit numeric value, convert the real value to a scaled integer, split it into two 16-bit registers, and write both registers with Function Code 16.

- To write a boolean or trigger, write value 0 or 1 to the single holding register. Trigger registers use value 1 and are cleared internally by the firmware after processing.

- To write the country, write 16 registers starting at CONFIGURATION Country. Each register contains two ASCII characters, high byte first.

- After changing persistent configuration values, write 1 to Save configuration so the firmware stores the new values in flash.

Request Groups

| Petition | Direction | Description |
| --- | --- | --- |
| STATUS | Receive / Read | Collects the current firmware values: position, angles, mode flags, state and country. The master normally polls this block. |
| CONFIGURATION | Send / Write | Writes persistent configuration values such as latitude, longitude, pan, tilt, tilt correction and country. Write SAVE = 1 to store them in flash. |
| MANUAL_MODE | Send / Write | Writes X/Z target values and trigger registers used to enter manual mode, submit the movement or leave manual mode. |
| MODO_EPH | Send / Write | Writes azimuth/elevation values and trigger registers used to calculate and submit the ephemeris-based movement. |
| AUTO_MODE | Send / Write | Writes a trigger register used to toggle automatic mode. |

Data Encoding

x100 means that the real decimal value is multiplied by 100 before being sent as an integer. Example: 12.34 degrees is sent as 1234. The firmware divides by 100 to recover 12.34.

x10000 is used for latitude and longitude to keep more decimal precision. Example: 40.4168 degrees is sent as 404168. The firmware divides by 10000 to recover 40.4168.

Signed 32-bit values occupy two holding registers, high word first. If the scaled value is 1234 decimal, it is sent as high word 0 and low word 1234.

ASCII strings use two characters per holding register, high byte first. Country is limited to 31 useful characters plus the trailing C string terminator.

Register Table

| Petition | Type | Variable | Offset | 40001 Address | Regs | Format | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| STATUS | Receive | Latitude | 0 | 40001 | 2 | int32, degrees x10000 | Read current latitude |
| STATUS | Receive | Longitude | 2 | 40003 | 2 | int32, degrees x10000 | Read current longitude |
| STATUS | Receive | Pan | 4 | 40005 | 2 | int32, degrees x100 | Read current pan angle |
| STATUS | Receive | Tilt | 6 | 40007 | 2 | int32, degrees x100 | Read current tilt angle |
| STATUS | Receive | Tilt_correction | 8 | 40009 | 1 | uint16 boolean | 0 = false, 1 = true |
| STATUS | Receive | g_x_val | 9 | 40010 | 2 | int32, mm x100 | Read current X position/value |
| STATUS | Receive | g_z_val | 11 | 40012 | 2 | int32, mm x100 | Read current Z position/value |
| STATUS | Receive | Azimuth | 13 | 40014 | 2 | int32, degrees x100 | Read current azimuth |
| STATUS | Receive | Elevation | 15 | 40016 | 2 | int32, degrees x100 | Read current elevation |
| STATUS | Receive | AOIL | 17 | 40018 | 2 | int32, degrees x100 | Read calculated AOIL |
| STATUS | Receive | AOIT | 19 | 40020 | 2 | int32, degrees x100 | Read calculated AOIT |
| STATUS | Receive | State | 21 | 40022 | 1 | uint16 enum | Firmware FSM state |
| STATUS | Receive | auto_on | 22 | 40023 | 1 | uint16 boolean | Automatic mode flag |
| STATUS | Receive | manual_time | 23 | 40024 | 1 | uint16 boolean | 0 = GPS time, 1 = manual time |
| STATUS | Receive | Country | 24 | 40025 | 16 | ASCII string | Same internal variable as configuration country, read-oriented mirror |
| CONFIGURATION | Send | Latitude | 100 | 40101 | 2 | int32, degrees x10000 | Write latitude configuration |
| CONFIGURATION | Send | Longitude | 102 | 40103 | 2 | int32, degrees x10000 | Write longitude configuration |
| CONFIGURATION | Send | Pan | 104 | 40105 | 2 | int32, degrees x100 | Write pan configuration |
| CONFIGURATION | Send | Tilt | 106 | 40107 | 2 | int32, degrees x100 | Write tilt configuration |
| CONFIGURATION | Send | Tilt_correction | 108 | 40109 | 1 | uint16 boolean | 0 = false, 1 = true |
| CONFIGURATION | Send | Country | 110 | 40111 | 16 | ASCII string | Write country name, max 31 chars plus null terminator |
| CONFIGURATION | Send | Save configuration | 126 | 40127 | 1 | trigger | Write 1 to save configuration to flash |
| MANUAL_MODE | Send | g_x_target | 140 | 40141 | 2 | int32, mm x100 | Write requested X target |
| MANUAL_MODE | Send | g_z_target | 142 | 40143 | 2 | int32, mm x100 | Write requested Z target |
| MANUAL_MODE | Send | begin_manual | 144 | 40145 | 1 | trigger | Write 1 to request MANUAL state |
| MANUAL_MODE | Send | submit_manual_goto | 145 | 40146 | 1 | trigger | Write 1 to move to X/Z target |
| MANUAL_MODE | Send | end_manual | 146 | 40147 | 1 | trigger | Write 1 to return to STDBY |
| MODO_EPH | Send | Azimuth | 150 | 40151 | 2 | int32, degrees x100 | Write ephemeris azimuth |
| MODO_EPH | Send | Elevation | 152 | 40153 | 2 | int32, degrees x100 | Write ephemeris elevation |
| MODO_EPH | Send | begin_eph_input | 154 | 40155 | 1 | trigger | Write 1 to request EPH_INPUT state |
| MODO_EPH | Send | submit_eph_input | 155 | 40156 | 1 | trigger | Write 1 to calculate target and move |
| MODO_EPH | Send | end_eph_input | 156 | 40157 | 1 | trigger | Write 1 to return to STDBY |
| AUTO_MODE | Send | toggle_auto_mode | 160 | 40161 | 1 | trigger | Write 1 to toggle AUTO_MODE |

Country Example

To write Spain into CONFIGURATION Country, write 16 registers starting at offset 110. The first registers are: 0x5370 for Sp, 0x6169 for ai, 0x6E00 for n and null terminator (‘\0’), then zeros until the 16-register block is complete.

Firmware Integration Note

The Modbus stack callback should call ModbusMap_ReadHoldingRegisters() for Function Code 03 and ModbusMap_WriteHoldingRegisters() for Function Code 06 or 16. The map layer then updates or reads the application globals and posts FSM events when trigger registers are written.